CMLS Project – JUCE PART (notes)

What we need to check :

Latency  The chain Arduino → OSC → JUCE → bus virtuel → SuperCollider  introduces many jumps. For live performance, we need to pay attention to buffer sizes at each step

Synchronization JUCE/SuperCollider — Both use their own audio clock. Virtual busses de-couple them properly but we need to check that sample rates are the same everywhere.

Main JUCE steps :

1. Project setup and audio routing Create an audio plugin or a standalone app with 2 inputs(guitar +micro) and 2 outputs (Supercollider via virtual bus). Setup AudioDeviceManager to take the good inputs/outputs

2. Noise Gate On each canal. Compute RMS over a moving window, comparison with threshold, gain with attack/release to avoid clicks

3. Compressor After noise gate, classic compressor : detection of level (peak or RMS), computation of gain reduction, smoothing with attack/release, makeupgain

4. Envelope Follower Extract dynamic envelope of each signal (guitar+voice) : rectification + lowpass filter. This signal will be used to feed OSC messages to Supercollider and graphic interface (ex: if we want to display the levels on GUI) (but actually maybe this part is useless)

6. Pitch detection Detection of the pitch of the guitar. For that we use the YIN medthod.

5. Module OSC (emission) Use juce::OSCSender to send in UDP the envelope follower data and receive juce::OSCReceiver for the data coming from arduino. Messages can be /envelope/guitar, /envelope/voice, /preset, /param/1, etc.

6. Latency tests + opti Measure latency from beginning to end with a test signal, adjust buffer sizes, be sure that thread audio never does memory allocations

Modules we should use :

juce_audio_basics, juce_audio_devices, juce_audio_processors, juce_dsp , juce_osc, juce_gui_basics, juce_gui_extra, juce_graphics, juce_core, juce_events, juce_audio_utils

Architecture :

- Arduino → JUCE :

/freeze         (int 0 or 1)

/blend          (float 0.0 to 1.0)

- JUCE → SuperCollider :

/pitch/guitar   (float : freq in Hz)   ← calculated by JUCE

/envelope/guitar (float : level)            ← calculated by JUCE

/envelope/voice  (float : level)            ← calculated by JUCE

/freeze         (int : retransmitted)           ← coming from Arduino

/blend          (float : retransmitted)         ← coming from Arduino

- Arduino → SuperCollider (directly) :

/heartbeat      (float : BPM)

GUI :

- Vumeter guitar and voice (level in real time) ? Or the blended signal ?

-  Visual signal (led) when presets change

- Compression indication ? (gain reduction)

- Heartbeat rate and visual change with respect to it (color of plugin shifts from blue to red ?)

- Value of potentiometer (blend indicator?)

How does it updates in real time ? :

Timer (30fps) : → sends OSC → calls repaint() → JUCE calls paint() → redraws everything

TESTS

| Test | Result | Expected | Verdict |
| --- | --- | --- | --- |
| Gate sweak signal | 0 | 0.0 (cut) | ✅ |
| Gate strong signal | 0.5 | 0.5 (pass) | ✅ |
| Compressor strong signal | 0.213 | < 0.8 (reduced) | ✅ |
| Compressor weak signal | 0.00267 | ≈ 0.01 (almost intact) | ✅ |
| YIN 440Hz | 440.018 Hz | 440 Hz | ✅ excellent |

YIN very good — error is 0.018 Hz, < 0.004%, very precise.

Additional notes

- How does a OSC message work ?

OSC message composed of 2 things :

/adress    value

Examples :

/preset         2

/param/threshold   0.75

The adress is like a path to identify the message type

In our project :

/preset corresponds to footswitch — digital sensor :

User press footswitch → Arduino sends "/preset" 2

→ JUCE loads preset 2

/param/threshold corresponds to analog sensor (ex: potentiometer) :

Utilisateur uses potentiometer → Arduino sends "/param/threshold" 0.75

→ JUCE updates gateThreshold

- Threads and Timer

What is a thread ?

A thread is a sequence of instructions that the processor executes. Program can have different threads running at the same time, each one with its own task.

In JUCE there are 2 main threads:

Main thread (UI)                    Audio thread

↓                                                ↓

- Designs interface                   - Calls getNextAudioBlock

- Deals with mouse clicks       - DSP

- Receives OSC messages     - Maximal priority

-  ~60fps                                         -  ~44100 samples/sec

Whis is the audio thread special ?

It is in real time – system gives him maximal prioritybecause it has to finish the work before the buffer is empty, otherwise you hear crackling noise

Thus, there are strict rules of what we can do inside :

Thread audio — AUTHORIZED ✅        Thread audio — FORBIDDEN ❌

Maths calculus                                               Memory allocation

Read/Write tables                                         Disk access

Operations on samples                              Network operations (OSC)

Network operations forbidden because they can block system an undetermined time — the time it leaves on the network. This would lead to missing the audio deadline -> cracklings

Example :

Thread audio (getNextAudioBlock) :

compute envGuitar = 0.75

oscSender.send(...)  ← ⚠️ network operation

can block 1ms, 5ms, 10ms...

meanwhile buffer empties..

→ CLICKS

The solution : Timer

Timer runs on the main thread (UI), not on audio thread. It calls its function timerCallback() at regularly spaced time instants :

In .cpp :

startTimerHz(30); // calls timerCallback() 30 times per second

Audio thread updates variables envGuitar et envVoice, and timer reads these variables and sends OSC peacefully on the main thread :

Thread audio :                                        Thread principal (Timer) :

computes envGuitar = 0.75          every 33ms :

computes envVoice  = 0.45          reads envGuitar → sends OSC

reads envVoice  → sends OSC

↑ quickly, no blocking                     ↑ can take the time to do it

Visually

Audio thread : |--DSP--|--DSP--|--DSP--|--DSP--|--DSP--|

Main thread : |-------Timer-------|-------Timer-------|

↑ sends OSC here without bothering the DSP

Last thing : Shared variables

2 threads write and read envGuitar at the same time (shared variable). In our case it is acceptable because :

•	It’s a simple float

•	In the worst case, we read a slightly off value in the time (not perceptible)

For more sensible data, we should use synchronization mechanisms, but for our use it is enough.

UPDATE 11/05 (Damiano):

- Integrated the work done by Marin with a circular buffer implementation in order to reduce latency from around 44ms to around 11ms.

- Updated GitHub Repo and created individual branches

Next Steps (see Gemini answer below for more details*):

- Damiano: practical test with guitar and vocals, fine tuning of gate and compressor parameters. Check if Pitch post-processing and pitch quantization are needed.

- Marin: Audio routing implementation with Virtual Audio Cable, bidirectional OSC protocol

After that: guitar DSP implementation and relative presets

*Gemini answer:
To complete the JUCE section and get it ready for integration with SuperCollider and Arduino, you need to transition from a "monolithic" architecture to a more modular and robust one.

Considering your backgrounds in engineering and music, the work can be divided into two main tracks: Signal Integrity & Analysis (the "Brain") and System Infrastructure & Communication (the "Muscle").

Here is the detailed breakdown of the next steps:

1. Workstream A: Signal Integrity & Analysis

This track focuses on ensuring that the data sent to SuperCollider is as clean and stable as possible.

- Refining the DSP Chain: Optimize the attack and release coefficients for the Gate and Compressor. You need to test how these react to different picking styles (staccato vs. legato) to prevent the pitch tracker from receiving harsh transients that could trigger octave errors.

- Pitch Post-processing: The YIN algorithm, while robust, can produce "jitter" (micro-fluctuations in frequency). You should implement a filter (e.g., a moving average or median filter) on the detectedPitch before transmitting it.

- Pitch Quantization (Optional but recommended): Implement logic that "anchors" the detected frequency to the nearest MIDI note if the signal confidence is high. Send both the raw and quantized values, allowing SuperCollider to decide whether to apply a smooth portamento or hard pitch steps.

2. Workstream B: System Infrastructure & Communication

This track focuses on the "transport" of data and audio signals across the different platforms.

- Audio Routing Implementation (Virtual Cable): Configure JUCE so that the output of channels 1 and 2 (the processed voice and guitar) is correctly routed to the virtual cable. This involves latency management: if JUCE introduces a processing delay, the OSC messages and the audio signal must still arrive at SC synchronously.

- Bidirectional OSC Protocol: Expand the incoming message handling. JUCE must receive sensor data (via Arduino/Processing) and determine what to process internally (e.g., changing a preset) and what to forward to SuperCollider (e.g., joystick values controlling chord complexity).

- State Management (Presets): Expand the Preset struct/class to include not only DSP parameters but also logical states (e.g., toggling between "Guitar Mode" and "Voice Mode").

- Proposed Division of Labor

- Given the structure of the project, here is how you and your colleague can divide the tasks to work in parallel effectively:

| Area | Tasks for Collab 1 (DSP/Music) | Tasks for Collab 2 (Systems/Dev) |
| --- | --- | --- |
| Audio | Gate/Comp calibration and dynamic guitar testing. | I/O configuration for Virtual Cable and buffer management. |
| Data | Pitch smoothing and filtering logic (YIN). | Full implementation of the OSC client/server protocol. |
| Integration | Defining the musical intervals for the presets. | Handling the messaging pipeline between Arduino and SC. |

Immediate Next Technical Step

The first crucial test is verifying that SuperCollider receives the audio signal from JUCE through the virtual cable without distortion or dropouts. Once this audio "bridge" is established and stable, you can safely proceed with sending OSC data alongside it.

Have you guys already installed a virtual audio routing software, or do you need recommendations for setting this up on Windows/macOS?